# AI_CONTEXT_FINAL.md Specification

## ROLE OF THIS DOCUMENT

This document is the ONLY authoritative specification for the MacroPad X firmware. Any AI (Codex/ChatGPT/etc.) must read it completely before editing code. Architecture is frozen. Existing public APIs must not be changed unless the user explicitly requests it.

## PROJECT SUMMARY

Project: MacroPad X

Hardware:

- Arduino Pro Micro (ATmega32U4, 16 MHz)

- GC9A01 round display (240x240, SPI)

- Mechanical encoder with push button (20 detents)

- 20-key mechanical keyboard

- 4x5 key matrix

- One 1N4148 diode per key

- Kailh hot swap sockets

- WS2812 RGB (future)

- USB HID

- Windows companion application (future)

Primary goals:

- Professional architecture

- Stable firmware

- Fast response

- Smooth UI

- Clean C++17 code

- Easy maintenance

- No unnecessary abstractions

## FINAL PINOUT

Encoder

D2 A

D3 B

D4 Button

Matrix Rows

D5

D6

D7

D8

Matrix Columns

D9

D14 (MISO)

A0

A1

D1 (TX)

Display

D10 CS

A3 DC

A2 RST

D15 SCK

D16 MOSI

RGB

D0

## PROJECT TREE

src/

Core/

Config.h

Colors.h

Pins.h

Types.h

Timer.h/.cpp

Drivers/

Display

Encoder

Keyboard

RGB

UI/

DisplayState

HomeScreen

SettingsScreen

Protocol/

SerialProtocol

main.cpp

## MODULE RESPONSIBILITIES

Core:

Compile-time constants, shared types, timers.

Display:

Graphics primitives ONLY.

Knows nothing about application logic.

Encoder:

Reads hardware only.

Returns EncoderState.

Keyboard:

Scans matrix only.

Returns KeyboardState.

RGB:

Controls LEDs only.

Protocol:

Communicates with PC.

Updates DisplayState only.

DisplayState:

Single source of truth.

HomeScreen:

Renders DisplayState through Display.

main.cpp:

Creates objects and connects modules together.

## PUBLIC APIS

Display:

begin()

update()

clear()

drawPixel()

drawLine()

drawCircle()

fillCircle()

drawRect()

fillRect()

drawText()

drawCenteredText()

Encoder:

begin()

update()

state()

Keyboard:

begin()

update()

state()

RGB:

begin()

update()

setBrightness()

SerialProtocol:

begin()

update()

HomeScreen:

draw(Display&, const DisplayState&)

## DISPLAY STATE

DisplayState stores:

mode

volume

muted

dirty

Every module may modify DisplayState.

Only UI renders DisplayState.

## CODING STANDARD

Language: C++17

Use:

constexpr

enum class

references

namespaces

fixed-size arrays

Never use:

delay()

String

malloc()

free()

new

delete

No dynamic allocation.

No blocking loops.

No global mutable state except shared application state objects if required.

## PERFORMANCE

Keyboard scan latency <5 ms

Encoder latency <5 ms

Display target ≈30 FPS

Keep RAM usage low (ATmega32U4).

## DEVELOPMENT ORDER

1. Keyboard

2. HomeScreen

3. SettingsScreen

4. Connect Encoder -> DisplayState

5. Connect Keyboard

6. USB HID

7. RGB

8. Windows companion

## RULES FOR CODEX

1. Read this document before changing code.

2. Analyze existing source first.

3. Preserve architecture.

4. Never rename public classes or methods.

5. Never move files.

6. Never replace PlatformIO.

7. Never introduce RTOS or heavy frameworks.

8. Modify existing modules instead of redesigning.

9. Return COMPLETE files when asked to modify a file.

10. Keep modules loosely coupled.

11. Prefer simple readable code over clever abstractions.

12. If uncertain, preserve current architecture rather than inventing a new one.

## PROJECT VISION

The final firmware should resemble a commercial macro pad:

- smooth graphics

- responsive encoder

- reliable keyboard scanning

- clean USB HID

- modular architecture

- easy future expansion without architectural rewrites.